Supporting Calculations

Matboard Bridge Project

CIV102

Contents

2	Design 0 Calculations	2

2.1	Hand Calculations and Code Output	2

2.1.1	Reaction Forces, SFD and BMD	2

2.1.2	Sectional Properties	3

2.1.3	Flexural Failure:	4

2.1.4	Matboard Shear Failure	5

2.1.5	Glue Shear Failure	6

2.1.6	Plate Buckling	7

2.1.7	Shear Buckling	8

2.1.8	Failure Load (Pfail)	9

2.1.9	Midspan Deflection	10

2.2	SFD and BMD Data Plots	10

2.2.1	Pfail SFD and BMD	10

2.2.2	SFD vs Material Shear and Shear Buckling Failures	10

2.2.3	BMD vs Material Moment and Buckling Failures	11

3	Appendices	12

3.1	Appendix I: MATLAB Code Parameter Names	12

3.2	Appendix II: Complete MATLAB Code:	13

# Design 0 Calculations

## Hand Calculations and Code Output

### Reaction Forces, SFD and BMD

Code Sample Call of Relevant Function(s):

load = -1;

[SFD_PL, BMD_PL, P] = ApplyPL(550, load, x, P, n); % Construct SFD, BMD

[SFD_PL, BMD_PL, P] = ApplyPL(L, load, x, P, n); % Construct SFD, BMD

SFD_PL and BMD_PL are array containing the value of the SFD and BMD at each x coordinate. The ApplyPL function itself does not plot the SFD/BMD, however, for the sake of clarity the SFD and BMD were plotted using the VisualizePL function using the arrays generated using the function above.

Code Output:

### Sectional Properties

Code Sample Call of Relevant Function(s):

[Y_bridge, I_bridge, Q_bridge, b_bridge, section_heights] = SectionProperties(xc, tfw, tft, hw, tw, ws, bfw, bft, rtw, rtt, rbw, rbt, n);

Code Output:

I_bridge = 415675.1027

Y_bridge = 41.7016

Q_bridge = 6248.3312

b_bridge = 2.54

section_heights = 75

The output of the function above are all arrays of length n (the length of the bridge) except for b_bridge and Q_bridge which are n by y matrices where y is the height of the cross section. The code output section only displayed one value from the array. However, for design 0 all the values within these arrays are constant since the cross section is consistent throughout the length of the bridge. As for Q_bridge and b_bridge, their values at the centroid were be displayed as they will be used in further calculations (see section 2.1.4) For further details on the function parameters above and their definitions see Appendix II.

### Flexural Failure:

Code Sample Call of Relevant Function(s):

[ m_mat_tension ] = MfailMatT( I_bridge, Y_bridge, section_heights, SigT, BMD_PL);

[ m_mat_compression ] = MfailMatC( I_bridge, Y_bridge, section_heights, SigC, BMD_PL );

Code Output:

m_mat_tension = 299043

m_mat_compression = -59808.6

The above functions output the matboard tension and compression failure moment values for each point on the bridge as an array of length n, where n is the length of the bridge. These functions automatically determine what side of the bridge experiences tension and compression based on where the BMD is negative or positive. For this reason, the arrays contain both the failure moment values for both the top and bottom flange according to the aforementioned condition. For the sake of comparison to the manual calculation seen above, the code output above only shows the m_mat_tension value for when the bottom flange is under tension. Similarly, to m_mat_tension, the value of m_mat_compression for when the bottom flange is under compression was shown.

### Matboard Shear Failure

Code Sample Call of Relevant Function(s):

[v_fail] = Vfail(I_bridge, b_bridge, Y_bridge, TauU, Q_bridge);

Code Output:

v_fail = 675.9181813

Since the function above outputs v_fail, an array of length n, where n is the length of the bridge, only one value of v_fail was shown in the sample output above.

### Glue Shear Failure

Code Sample Call of Relevant Function(s):

[ V_GlueTF V_GlueBF V_GlueTW V_GlueBW] = VglueFail(I_bridge, Q_bridge, b_bridge, TauG, tft, bft, section_heights, xc);

Code Output:

V_GlueTF = NaN  Top flange layer joint

V_GlueBF = NaN  Bottom flange layer joint

V_GlueTW = 4714.769465  Top flange/web joint

V_GlueBW = 557.7619034  Bottom flange/web joint

Note: The reason for the difference between the code and the and the hand calculations above is due to the lack of millimeter precision in the Q value used in calculating the above values in the function due to the limitations caused by storing Q in a matrix. For this reason, a significant factor of safety was used when considering the various glue failure loads in our final bridge design due to these inaccuracies.

### Plate Buckling

Code Sample Call of Relevant Function(s):

[ M_Buck1, M_Buck2, M_Buck3 ] = MfailBuck( xc, bfw, bft, tfw, tft, ws, tw, section_heights, Y_bridge,

Code Output:

M_Buck1 (BMD > 0) = 46000.3  Plate #1

M_Buck1 (BMD < 0) = -36730.9  Plate #6

M_Buck2 (BMD > 0) = 293254  Plates #2 and #3

M_Buck3 (BMD > 0) = 403585  Plates #4 and #5

M_Buck3 (BMD < 0) = -202226  Plates #7 and #8

Note: All outputs of MfailBuck are arrays of length n. Only 2 values for each are shown above; one when the BMD is positive and the other when the BMD is negative. Note the lack of a value for M_Buck2 when the BMD is negative. This is due to the lack of a ‘overhang’ on the bottom flange meaning there is no buckling of this kind when the BMD is negative since the bottom is in compression.

### Shear Buckling

Code Sample Call of Relevant Function(s):

[ v_buck ] = VfailBuck(xc, tw, a, hw, E, mu, n, I_bridge, b_bridge, Y_bridge, Q_bridge);

Code Output:

v_buck(0 < x < 550) = 904.889  Section 1

v_buck(550 < x < 1060) = 907.406  Section 2

v_buck(1060 < x < 1250) = 1018.81  Section 3

Note: The output of VfailBuck is an array of length n. The values shown above represent the values of v_buck within the specified intervals.

### Failure Load (Pfail)

Code Sample Call of Relevant Function(s):

[ Pf, failure_mode ] = FailLoad( load, SFD_PL, BMD_PL, v_fail, v_buck, m_mat_tension, m_mat_compression, M_Buck1, M_Buck2, M_Buck3, V_GlueTF, V_GlueBF, V_GlueTW, V_GlueBW, true, "Design0");

Code Output:

Pf = 193.0605836

Failure_mode = Center Flange Buckling

The difference in Pf and Pfail in hand calculations can likely be attributed to a rounding error. Additionally, for further details on function parameters, see Appendix II code comments.

### Midspan Deflection

ADD HERE

## SFD and BMD Data Plots

The following section contains the plots produced by plotting the arrays generated by the functions discussed in section 3.1 using the VisualizePL function. For details on this function see comments in Appendix II.

### Pfail SFD and BMD

### SFD vs Material Shear and Shear Buckling Failures

### BMD vs Material Moment and Buckling Failures

# Appendices

## Appendix I: MATLAB Code Parameters

| Name | Purpose | Type |
| --- | --- | --- |
| n | Determines the number of locations to evaluate bridge failure | Integer |
| load | Stores value of point load to be applied | Integer |
| x | Defines x coordinates | Array |
| P | Stores point loads | Array |
| xc | Stores x coordinates of where cross section changes | Array |
| tfw | Stores top flange width | Array |
| tft | Stores top flange thickness | Array |
| hw | Stores web height | Array |
| tw | Stores web thickness | Array |
| bfw | Stores bottom flange width | Array |
| bft | Stores bottom flange thickness | Array |
| a | Stores x coordinates of diaphragms | Array |
| ws | Stores space between webs | Array |
| rtw | Stores width of top flange/web glue tab | Array |
| rtt | Stores thickness of top flange/web glue tab | Array |
| rbw | Stores width of bottom flange/web glue tab | Array |
| rbt | Stores thickness of bottom flange/web glue tab | Array |
| SigT | Stores matboard tensile strength | Integer |
| SigC | Stores matboard compressive strength | Integer |
| E | Stores young’s modulus of matboard | Integer |
| TauU | Stores shear stress of matboard | Integer |
| TauG | Stores shear stress of glue | Integer |
| mu | Stores poisson’s ratio | Integer |
| Y_bridge | Stores centroid for each x coordinate of bridge | Array |
| I_bridge | Stores second moment of area for each x coordinate of bridge | Array |
| Q_bridge | Stores first moment of area for each x and y coordinate of bridge | Matrix |
| b_bridge | Stores thickness for each x and y coordinate of bridge | Matrix |
| section_heights | Stores height of bridge for each x coordinate | Array |
| v_fail | Stores shear failure value for each x coordinate | Array |
| v_buck | Stores shear buckling failure value for each x coordinate | Array |
| m_mat_tension | Stores moment tension failure value for each x coordinate | Array |
| m_mat_compression | Stores moment compression failure value for each x coordinate | Array |
| m_Buck1 | Stores moment plate buckling failure values for center of top or bottom flange (based on BMD sign) | Array |
| m_Buck2 | Stores moment plate buckling failure values for overhang of top or bottom flange (based on BMD sign) | Array |
| m_Buck3 | Stores moment plate buckling failure values for webs | Array |
| V_GlueTF | Stores shear failure value for top flange glue | Array |
| V_GlueBF | Stores shear failure value for bottom flange glue | Array |
| V_GlueTW | Stores shear failure value for top flange/web joint glue | Array |
| V_GlueBW | Stores shear failure value for bottom flange/web joint glue | Array |

## Appendix II: Complete MATLAB Code:

%% 0. Initialize Parameters

n = 1250; % Number of locations to evaluate bridge failure

L = 1250; % Length of bridge

x = linspace(0, L, n); % Define x coordinate

P = zeros(1,n); % Initializes Loads

%% 1. Point Loading Analysis (SFD, BMD)

load = -1062;

[SFD_PL, BMD_PL, P] = ApplyPL(550, load, x, P, n); % Construct SFD, BMD

[SFD_PL, BMD_PL, P] = ApplyPL(L, load, x, P, n); % Construct SFD, BMD

xc = [0 550 1060 L]; % Location, x, of cross-section change

tfw = [100 100 100 100]; % Top Flange Width

tft = [1.27 1.27 1.27 1.27]; % Top Flange Thickness

hw = [72.46 72.46 72.46 72.46]; % Web Height

tw = [1.27 1.27 1.27 1.27]; % Web Thickness (Assuming 2 separate webs)

bfw = [80 80 80 80]; % Bottom Flange Width

bft = [1.27 1.27 1.27 1.27]; % Bottom Flange Thickness

a = [0 550 1060 L]; % Diaphragm x coords

ws = [77.46 77.46 77.46 77.46]; % Web Spacing

rtw = [10 10 10 10]; % little rectangles at top of webs for glue area width

rtt = [1.27 1.27 1.27 1.27]; % little rectangles at top of webs for glue area thickness

rbw = [0 0 0 0]; % little rectangles at bottom of webs for glue area width

rbt = [0 0 0 0]; % little rectangles at bottom of webs for glue area thickness

% Optional but you need to ensure that your geometric inputs are correctly implemented

VisualizeBridge(xc, tfw, tft, hw, tw, ws, bfw, bft, rtw, rtt, rbw, rbt);

%% 3. Define Material Properties

SigT = 30;

SigC = 6;

E = 4000;

TauU = 4;

TauG = 2;

mu = 0.2;

% Calls Function to calculate geometric properties (output is in the form of arrays)

[Y_bridge, I_bridge, Q_bridge, b_bridge, section_heights] = SectionProperties(xc, tfw, tft, hw, tw, ws, bfw, bft, rtw, rtt, rbw, rbt, n );

% Outputs Calculations from above function for each cross section

for c = 1:length(xc)

if xc(c) ~=0

"Y bar cross section " + n + " = " + Y_bridge(xc(c))

"I for cross section " + n + " = " + I_bridge(xc(c))

"Q for cross section at centriod " + n + " = " + Q_bridge(xc(c), round(Y_bridge(xc(c)))) % Note this value is rounded due to mm precision in Q matrix

"Height of cross section " + n + " = " + section_heights(xc(c))

end

end

[v_fail] = Vfail(I_bridge, b_bridge, Y_bridge, TauU, Q_bridge);

[ v_buck ] = VfailBuck(xc, tw, a, hw, E, mu, n, I_bridge, b_bridge, Y_bridge, Q_bridge);

[ m_mat_tension ] = MfailMatT( I_bridge, Y_bridge, section_heights, SigT, BMD_PL);

[ m_mat_compression ] = MfailMatC( I_bridge, Y_bridge, section_heights, SigC, BMD_PL );

[ M_Buck1, M_Buck2, M_Buck3 ] = MfailBuck( xc, bfw, bft, tfw, tft, ws, tw, section_heights, Y_bridge, I_bridge, E, mu, BMD_PL);

[ V_GlueTF V_GlueBF V_GlueTW V_GlueBW] = VglueFail(I_bridge, Q_bridge, b_bridge, TauG, tft, bft, section_heights, xc);

% FAILURE TYPES FOR FailLoad FUNCTION:

% matt = moment matboard tension failure

% matc = moment matboard compression failure

% buck1 = case 1 buckling of top or bottom flange (center part between webs)

% buck2 = case 2 buckling of top or bottom flange (part that sticks out past the webs)

% buck3 = case 3 buckling of left and right webs

% vmat = matboard shear failure

% vbuck = matboard shear buckling failure

% vgluetf = shear glue failure of top flange

% vgluebf = shear glue failure of bottom flange

% vgluetw = shear glue failure of top flange/web connection

% vgluebw = shear glue failure of bottom flange/web connection

% Calls function to calculate failure load and failure mode

[ Pf, failure_mode ] = FailLoad( load, SFD_PL, BMD_PL, v_fail, v_buck, m_mat_tension, m_mat_compression, M_Buck1, M_Buck2, M_Buck3, V_GlueTF, V_GlueBF, V_GlueTW, V_GlueBW, true, "Design0");

VisulizePL(x, load, Pf, SFD_PL, BMD_PL, v_fail, v_buck, m_mat_tension, m_mat_compression, M_Buck1, M_Buck2, M_Buck3, V_GlueTF, V_GlueBF, V_GlueTW, V_GlueBW);

% Outputs Failure load and fail type

"Pfail = " + Pf

"Failure mode = " + failure_mode

% Calls function to check if design overuses material

[ material_ok ] = MaterialCheck(xc, tfw, tft, hw, tw, ws, bfw, bft, rtw, rtt, rbw, rbt);

% Outputs Material Check results

"Material is: " +  material_ok

%%

function [ SFD, BMD, Loads ] = ApplyPL( xP, P, x, Loads, n )

% Constructs SFD and BMD from application of 1 Point Load. Assumes fixed location of supports

%   Input: location and magnitude of point load. The previous Loads can be entered as input to

%       construct SFD of multiple point loads

%   Output: SFD, BMD, Loads, all 1-D arrays of length n

Loads(xP) = P;

moment = 0;

force = 0;

% Calculates sum of forces and moments for use in reaction force

% calculations

for i = 1:length(Loads)

if and(i ~= 1060, i ~= 1)

moment = Loads(i) * i + moment;

force = Loads(i) + force;

end

end

% calculates reaction forces

b = -moment/1060;

a = -b - force;

% Adds reaction forces to Loads array

Loads(1) = a;

Loads(1060) = b;

% Initializes new SFD

SFD = zeros(1, n);

% Calculates shear force at every point of bridge

shear = 0;

for i = 1:length(Loads)

if Loads(i) ~= 0

shear = shear + Loads(i);

end

SFD(i) = shear;

end

% Integrates SFD

BMD = cumtrapz(x, SFD);

end

%%

function [ ] = VisualizeBridge( csc, tfw, tft, wh, wt, ws, bfw, bft, rw, rh, rbw, rbh)

% Provides a graphical interpretation of user geometric inputs

%    Input: cross section changes x cordinates, top flange width, top flange thinckness, web height, web thickness, web spacing,

%        bottom flange width, bottom flange thickness, etc...

%    Output: plots polygons of crossections

for i = 1:length(csc)

% Calculates basic coordinates of features

y_top_flang_top_cord = bft(i) + wh(i) + tft(i);

y_top_flang_bot_cord = y_top_flang_top_cord - tft(i);

x_right_web_cord = tfw(i) - ((tfw(i) - ws(i)) / 2) + wt(i);

x_left_web_cord = ((tfw(i) - ws(i)) / 2) - wt(i);

x_left_bot_flang_cord = ((tfw(i) - bfw(i)) / 2);

x_right_bot_flang_cord = tfw(i) - ((tfw(i) - bfw(i)) / 2);

outside_x_cords = [x_left_bot_flang_cord x_left_bot_flang_cord x_left_web_cord x_left_web_cord 0 0 tfw(i) tfw(i) x_right_web_cord x_right_web_cord x_right_bot_flang_cord x_right_bot_flang_cord ];

outside_y_cords = [0 bft(i) bft(i) y_top_flang_bot_cord y_top_flang_bot_cord y_top_flang_top_cord y_top_flang_top_cord y_top_flang_bot_cord y_top_flang_bot_cord bft(i) bft(i) 0];

x_left_inside_web = x_left_web_cord + wt(i);

x_right_inside_web = x_right_web_cord - wt(i);

x_left_top_rectangle = x_left_inside_web + rw(i);

x_right_top_rectangle = x_right_inside_web - rw(i);

x_left_bot_rectangle = x_left_inside_web + rbw(i);

x_right_bot_rectangle = x_right_inside_web - rbw(i);

y_rectangle_top = y_top_flang_bot_cord - rh(i);

y_rectangle_bot = bft(i) + rbh(i);

inside_x_cords = [x_left_bot_rectangle x_left_bot_rectangle x_left_inside_web x_left_inside_web x_left_top_rectangle x_left_top_rectangle x_right_top_rectangle x_right_top_rectangle x_right_inside_web x_right_inside_web x_right_bot_rectangle x_right_bot_rectangle];

inside_y_cords = [bft(i) y_rectangle_bot y_rectangle_bot y_rectangle_top y_rectangle_top y_top_flang_bot_cord y_top_flang_bot_cord y_rectangle_top y_rectangle_top y_rectangle_bot y_rectangle_bot bft(i)];

% Draws rectangles for features

cross_section_shape = polyshape(outside_x_cords, outside_y_cords);

cross_section_shape = addboundary(cross_section_shape, inside_x_cords, inside_y_cords);

% Plots polygons

subplot(ceil(length(csc)/ 2), ceil(length(csc)/ 2), i)

plot(cross_section_shape)

% Labels Plots

if i ~= 1

title("Cross Section from x = " + csc(i-1) + " to " + csc(i))

else

title("Cross Section at x = 0")

end

axis equal

end

end

function out = getVarName(var)

out = inputname(1);

end

%%

function [ Y_bar, I, Q, b, heights ] = SectionProperties( csc, tfw, tft, wh, wt, ws, bfw, bft, rtw, rtt, rbw, rbt, n )

% Calculates important sectional properties. Including but not limited to ybar, I, Q, etc.

%    Input: cross section changes x cordinates, top flange width, top flange thinckness, web height, web thickness, web spacing,

%        bottom flange width, bottom flange thickness, etc

% Output: Sectional Properties at every value of x. Each property is a 1-D array of length n with the exception of b and Q which are n * max_height matrices.

Y_bar = zeros(1,n);

I = zeros(1, n);

for i = 1:length(csc)-1

heights(csc(i)+1:csc(i+1)) = (tft(i) + wh(i) + bft(i));

end

max_height = ceil(max(heights));

Q = zeros(n, max_height);

b = zeros(n, max_height);

for i = 1:length(csc)

% Areas for features

A_top = tfw(i) * tft(i);

A_webs = 2 * wt(i) * wh(i);

A_bot = bft(i) * bfw(i);

A_r_top = 2 * (rtw(i) * rtt(i));

A_r_bot = 2 * (rbw(i) * rbt(i));

% Local centriods for features

Y_top = (tft(i)/2 + wh(i) + bft(i));

Y_webs = (wh(i)/2 + bft(i));

Y_bot = (bft(i)/2);

Y_r_top = (wh(i) + bft(i) - rtt(i)/2);

Y_r_bot = (bft(i) + rbt(i)/2);

% I for features

I_top = (tfw(i) * (tft(i) ^ 3)) /12;

I_webs = 2 * (wt(i) * (wh(i) ^ 3)) /12;

I_bot = (bfw(i) * (bft(i) ^ 3)) /12;

I_r_top = 2 * (rtw(i) * (rtt(i) ^ 3)) /12;

I_r_bot = 2 * (rbw(i) * (rbt(i) ^ 3)) /12;

% Calculates I and Y for cross-section

Y_section = ((A_top * Y_top) + (A_webs * Y_webs) + (A_bot * Y_bot ) + (A_r_top * Y_r_top) + (A_r_bot * Y_r_bot)) / (A_bot + A_top + A_webs + A_r_bot + A_r_top);

I_section = (I_top + (A_top * ((Y_section - Y_top) ^ 2))) + (I_webs + (A_webs * ((Y_section - Y_webs) ^ 2))) + (I_bot + (A_bot * ((Y_section - Y_bot) ^ 2))) + (I_r_top + (A_r_top * ((Y_section - Y_r_top) ^ 2))) + (I_r_bot + (A_r_bot * ((Y_section - Y_r_bot) ^ 2)));

% Calculates Q, Adds I, Q and Y_bar for every x value of cross-section

if i ~= length(csc)

for j = csc(i):(csc(i+1)-1)

Y_bar(j+1) = Y_section;

I(j+1) = I_section;

for y = 1:((tft(i) + wh(i) + bft(i)));

if y <= bft(i)

b(j+1, y) = bft(i);

sub_area = y * bfw(i);

sub_centriod = (bft(i) - y)/2;

d = abs(Y_bar(csc(i) + 1) - sub_centriod);

Q(j+1, y) = sub_area * d;

elseif bft(i) < y && y <= (bft(i) + rbt(i))

b(j+1, y) = 2 * (rbw(i) + wt(i));

web_height = y - bft(i);

sub_area = A_bot + (2 * web_height  * (wt(i) + rbw(i)));

sub_centriod = ((A_bot * Y_bot) + ((2 * web_height * wt(i)) * ((web_height / 2) + bft(i)))) / sub_area;

d = abs(Y_bar(csc(i) + 1) - sub_centriod);

Q(j+1, y) = sub_area * d;

elseif (bft(i) + rbt(i)) < y && y <= (bft(i) + wh(i) - rtt(i))

b(j+1, y) = 2 * wt(i);

web_height = y - bft(i) - rbt(i);

sub_area = A_bot + (2 * web_height  * wt(i)) + A_r_bot;

sub_centriod = ((A_bot * Y_bot) + ((2 * web_height * wt(i)) * ((web_height / 2) + bft(i) + rbt(i))) + (A_r_bot * Y_r_bot)) / sub_area;

d = abs(Y_bar(csc(i) + 1) - sub_centriod);

Q(j+1, y) = sub_area * d;

elseif  (bft(i) + wh(i) - rtt(i)) < y && y <= (bft(i) + wh(i))

b(j+1, y) = 2 * (rtw(i) + wt(i));

web_height = y - wh(i) + rtt(i) - bft(i);

main_web_area =  (2 * (wh(i) - rtt(i)) * wt(i));

sub_area = A_bot + (2 * web_height  * (wt(i) + rtw(i))) + A_r_bot + main_web_area;

sub_centriod = ((A_bot * Y_bot) + (2 * web_height * (wt(i) + rtw(i)) * ((web_height / 2) + bft(i) + (wh(i) - rtt(i)))) + (A_r_bot * Y_r_bot) + (main_web_area * (((wh(i) - rtt(i))/2) + bft(i)))) / sub_area;

d = abs(Y_bar(csc(i) + 1) - sub_centriod);

Q(j+1, y) = sub_area * d;

else

b(j+1, y) = tfw(i);

top_height = y - wh(i) - bft(i);

sub_area = A_bot + A_webs + (top_height * tfw(i)) + A_r_bot + A_r_top;

sub_centriod = ((A_bot * Y_bot) + (A_webs * Y_webs) + (A_r_top * Y_r_top) + (A_r_bot * Y_r_bot) + (top_height * tfw(i) * ((top_height / 2) + wh(i) + bft(i)))) / sub_area;

d = abs(Y_bar(csc(i) + 1) - sub_centriod);

Q(j+1, y) = sub_area * d;

end

end

end

else

for j = csc(i):(n)

Y_bar(j) = Y_section;

I(j) = I_section;

for y = 1:((tft(i) + wh(i) + bft(i)))

if y <= bft(i)

b(j, y) = bft(i);

sub_area = y * bfw(i);

sub_centriod = (bft(i) - y)/2;

d = abs(Y_bar(csc(i)) - sub_centriod);

Q(j, y) = sub_area * d;

elseif bft(i) < y && y <= (bft(i) + rbt(i))

b(j, y) = 2 * (rbw(i) + wt(i));

web_height = y - bft(i);

sub_area = A_bot + (2 * web_height  * (wt(i) + rbw(i)));

sub_centriod = ((A_bot * Y_bot) + ((2 * web_height * wt(i)) * ((web_height / 2) + bft(i)))) / sub_area;

d = abs(Y_bar(csc(i)) - sub_centriod);

Q(j, y) = sub_area * d;

elseif (bft(i) + rbt(i)) < y && y <= (bft(i) + wh(i) - rtt(i))

b(j, y) = 2 * wt(i);

web_height = y - bft(i) - rbt(i);

sub_area = A_bot + (2 * web_height  * wt(i)) + A_r_bot;

sub_centriod = ((A_bot * Y_bot) + ((2 * web_height * wt(i)) * ((web_height / 2) + bft(i) + rbt(i))) + (A_r_bot * Y_r_bot)) / sub_area;

d = abs(Y_bar(csc(i)) - sub_centriod);

Q(j, y) = sub_area * d;

elseif  (bft(i) + wh(i) - rtt(i)) < y && y <= (bft(i) + wh(i))

b(j, y) = 2 * (rtw(i) + wt(i));

web_height = y - wh(i) + rtt(i) - bft(i);

main_web_area =  (2 * (wh(i) - rtt(i)) * wt(i));

sub_area = A_bot + (2 * web_height  * (wt(i) + rtw(i))) + A_r_bot + main_web_area;

sub_centriod = ((A_bot * Y_bot) + (2 * web_height * (wt(i) + rtw(i)) * ((web_height / 2) + bft(i) + (wh(i) - rtt(i)))) + (A_r_bot * Y_r_bot) + (main_web_area * (((wh(i) - rtt(i))/2) + bft(i)))) / sub_area;

d = abs(Y_bar(csc(i)) - sub_centriod);

Q(j, y) = sub_area * d;

else

b(j, y) = tfw(i);

top_height = y - wh(i) - bft(i);

sub_area = A_bot + A_webs + (top_height * tfw(i)) + A_r_bot + A_r_top;

sub_centriod = ((A_bot * Y_bot) + (A_webs * Y_webs) + (A_r_top * Y_r_top) + (A_r_bot * Y_r_bot) + (top_height * tfw(i) * ((top_height / 2) + wh(i) + bft(i)))) / sub_area;

d = abs(Y_bar(csc(i)) - sub_centriod);

Q(j, y) = sub_area * d;

end

end

end

end

end

end

function [ V_fail ] = Vfail( I, b, Y_bar, TauU, Q)

% Calculates shear forces at every value of x that would cause a matboard shear failure

%   Input: Sectional Properties (list of 1-D arrays), TauU (scalar material property)

%   Output: V_fail a 1-D array of length n

Qcent = zeros(1,length(Y_bar));

bcent = zeros(1,length(Y_bar));

for i = 1:length(Y_bar)

Qcent(i) = Q(i, round(Y_bar(i)));

bcent(i) = b(i, round(Y_bar(i)));

end

V_fail = TauU .* I .* bcent ./ Qcent;

end

function [ V_Buck ] = VfailBuck(csc, wt, ds, wh, E, mu, n, I, b, Y_bar, Q)

% Calculates shear forces at every value of x that would cause a shear buckling failure in the web

%   Input: Sectional Properties (list of 1-D arrays), E, mu (material property), etc...

%   Output: V_Buck a 1-D array of length n

factor = (5 * (pi ^ 2) * E ) / (12 * (1 - (mu ^ 2) ));

a_values = zeros(1, n);

web_thicknesses = zeros(1, n);

web_heights = zeros(1, n);

% Finds web thicknesses, web heights and a values for cross sections

for i = 1:(length(csc)-1)

web_thicknesses((csc(i)+1):(csc(i+1))) = wt(i);

web_heights((csc(i)+1):(csc(i+1))) = wh(i);

a_values((csc(i)+1):(csc(i+1))) = ds(i+1) - ds(i);

end

% Finds TauCrit

TauCrit = factor .* (((web_thicknesses ./ web_heights) .^ 2) + ((web_thicknesses ./ a_values) .^ 2));

% Calls V_fail function using tau crit value above

[ V_Buck ] = Vfail( I, b, Y_bar, TauCrit, Q);

end

function [] = VisulizePL(x, P, Pfail, SFD, BMD, V_fail, V_buck, M_MatT, M_MatC, M_Buck1, M_Buck2, M_Buck3, V_GlueTF, V_GlueBF, V_GlueTW, V_GlueBW)

% Plots all graphs (i.e BMD, SFD, Failure loads etc...)

%   Inputs: All failure condition loads as arrays of length n, SFD, BMD,

%   Outputs: Plots of above values/arrays

figure('WindowState', 'maximized')

% Plots SFD for Pfail and lables

subplot(2,3,1);

hold on;

plot(x, SFD);

set(gca, 'XAxisLocation', 'bottom', 'YAxisLocation', 'origin');

title(("SFD from P = " + P + "N, Pfail = " +  Pfail + "N"))

xlim([0,x(end)])

grid on;

grid minor

box on;

yline(0)

xlabel('Position on Bridge (mm)')

ylabel('Shear Force (N)')

% Plots BMD vs V_buck and lables

subplot(2,3,2);

hold on;

plot(x, SFD);

plot(x, V_fail(V_fail~=0), 'r-');

plot(x, -V_fail(V_fail~=0), 'r-');

plot(x, V_GlueTF(V_GlueTF~=0), 'g-')

plot(x, -V_GlueTF(V_GlueTF~=0), 'g-')

plot(x, V_GlueBF(V_GlueBF~=0), 'b-')

plot(x, -V_GlueBF(V_GlueBF~=0), 'b-')

plot(x, V_GlueTW(V_GlueTW~=0), 'm-')

plot(x, -V_GlueTW(V_GlueTW~=0), 'm-')

plot(x, V_GlueBW(V_GlueBW~=0), 'c-')

plot(x, -V_GlueBW(V_GlueBW~=0), 'c-')

set(gca, 'XAxisLocation', 'bottom', 'YAxisLocation', 'origin');

title('SFD vs Material Shear Failure')

xlim([0,x(end)])

grid on;

grid minor

box on;

ylim([-max(V_fail) - 100, max(V_fail) + 100])

yline(0)

legend('', '', 'Matboard Shear Failure', '', 'Glue Shear (TF) Failure', '',  'Glue Shear (BF) Failure', '', 'Glue Shear (TW) Failure', '', 'Glue Shear (BW) Failure', 'FontSize', 7)

xlabel('Position on Bridge (mm)')

ylabel('Shear Force (N)')

% Plots SFD vs V_buck and lables

subplot(2,3,3);

hold on;

plot(x, SFD);

plot(x, V_buck, 'r-');

plot(x, -V_buck, 'r-');

set(gca, 'XAxisLocation', 'bottom', 'YAxisLocation', 'origin');

title('SFD vs Shear Buckling Failure')

xlim([0,x(end)]);

ylim([-max(V_buck) - 100, max(V_buck) + 100]);

yline(0);

legend('', '', 'Web Shear Buckling Failure', '', 'FontSize', 7)

xlabel('Position on Bridge (mm)')

ylabel('Shear Force (N)')

grid on;

grid minor;

box on;

% Plots BMD for Pfail and lables

subplot(2,3,4);

hold on;

plot(x, BMD);

set(gca, 'XAxisLocation', 'bottom', 'YAxisLocation', 'origin', 'YDir', 'reverse');

title("BMD from P = " + P + "N, Pfail = " +  Pfail + "N")

xlim([0,x(end)])

grid on;

grid minor

box on;

yline(0)

xlabel('Position on Bridge (mm)')

ylabel('Bending Moment (Nmm)')

% Plots BMD vs Material Moment Failures and lables

subplot(2,3,5);

hold on;

plot(x, BMD);

plot(x, M_MatT)

plot(x, M_MatC)

set(gca, 'XAxisLocation', 'bottom', 'YAxisLocation', 'origin', 'YDir', 'reverse');

title('BMD vs Material Moment Failures ')

xlim([0,x(end)])

grid on;

grid minor

box on;

yline(0)

legend('', 'Matboard Tension Failure', 'Matboard Compression Failure', '', 'Location', 'Northwest', 'FontSize', 7)

xlabel('Position on Bridge (mm)')

ylabel('Bending Moment (Nmm)')

% Plots Moment Buckling and lables

subplot(2,3,6);

hold on;

plot(x, BMD);

plot(x, M_Buck1);

plot(x, M_Buck2);

plot(x, M_Buck3);

set(gca, 'XAxisLocation', 'bottom', 'YAxisLocation', 'origin', 'YDir', 'reverse');

title('BMD vs Material Buckling Failures ')

xlim([0,x(end)])

grid on;

grid minor

box on;

yline(0)

legend({'', 'Mid Flange Buckling', 'Side Flange Buclking', 'Web Compression Buckling', ''}, 'Location', 'Northwest', 'FontSize', 7)

xlabel('Position on Bridge (mm)')

ylabel('Bending Moment (Nmm)')

end

function [ M_MatT ] = MfailMatT( I, Y_bar, heights, SigT, BMD )

% Calculates bending moments at every value of x that would cause a matboard tension failure

%   Input: Sectional Properties (list of 1-D arrays), SigT (material property), BMD (1-D array), etc...

%   Output: M_MatT a 1-D array of length n

for i = 1 : length(BMD)

if BMD(i) > 0 % If the moment is positive, the tension failure will be at the bottom

M_MatT(i) = SigT * I(i) / (Y_bar(i));

elseif BMD(i) < 0 % If the moment is negative, the tension failure will be at the top

M_MatT(i) = -SigT * I(i) / (heights(i) - Y_bar(i));

end

end

end

function [ M_MatC ] = MfailMatC( I, Y_bar, heights, SigC, BMD ) % Similar to MfailMatT

% Calculates bending moments at every value of x that would cause a matboard Compression failure

%   Input: Sectional Properties (list of 1-D arrays), SigC (material property), BMD (1-D array), etc...

%   Output: M_MatC a 1-D array of length n

for i = 1 : length(BMD)

if BMD(i) > 0 % If the moment is positive, the compression failure will be at the top

M_MatC(i) = SigC * I(i) / (heights(i) - Y_bar(i));

elseif BMD(i) < 0 % If the moment is negative, the compression failure will be at the bottom

M_MatC(i) = -SigC * I(i) / Y_bar(i);

end

end

end

function [ M_Buck1 M_Buck2 M_Buck3 ] = MfailBuck( csc, bfw, bft, tfw, tft, ws, wt, heights, Y_bar, I, E, mu, BMD)

% Calculates bending moments at every value of x that would cause a buckling failure

% Input: Sectional Properties (list of 1-D arrays), E, mu (material property), BMD (1-D array)

% Output: M_Buck1, M_Buck2, M_Buck3 each 1-D array of length n

for c = 1:3

% Case 1 buckling

if c == 1

factor = (4 * (pi ^ 2) * E) / (12 * (1 - (mu ^ 2)));

for i = 1:length(BMD);

z = find(csc <= i, 1, 'last');

if BMD(i) < 0 % if moment negative, compression on bottom

t = bft(z);

y = - Y_bar(i);

b =  ws(z);

M_Buck1(i) = (factor * ((t / b) ^ 2)) * I(i) / y;

elseif BMD(i) > 0 % if moment positive, compression on top

t = tft(z);

y = heights(i) - Y_bar(i);

b = ws(z);

M_Buck1(i) = (factor * ((t / b) ^ 2)) * I(i) / y;

end

end

% Case 2 buckling

elseif c == 2

factor = (0.425 * (pi ^ 2) * E) / (12 * (1 - (mu ^ 2)));

for i = 1:length(BMD);

z = find(csc <= i, 1, 'last');

if BMD(i) < 0 % if moment negative, compression on bottom

t = bft(z);

y = - Y_bar(i);

b = (bfw(z) - (2 * wt(z)) - ws(z)) / 2;

if b ~= 0 && bfw(z) ~= 0

M_Buck2(i) = (factor * ((t / b) ^ 2)) * I(i) / y;

else

M_Buck2(i) = 0;

end

elseif BMD(i) > 0 % if moment positive, compression on top

t = tft(z);

y = heights(i) - Y_bar(i);

b = (tfw(z) - ws(z) - (2 * wt(z))) / 2;

if b ~= 0 && tfw(z) ~= 0

M_Buck2(i) = (factor * ((t / b) ^ 2)) * I(i) / y;

else

M_Buck2(i) = 0;

end

end

end

% Case 3 Buckling

else

factor = (6 * (pi ^ 2) * E) / (12 * (1 - (mu ^ 2)));

for i = 1:length(BMD);

z = find(csc <= i, 1,'last');

if BMD(i) < 0 % if moment negative, compression on bottom

t = wt(z);

y = - Y_bar(i);

b = abs(y) - bft(z);

if b ~= 0

M_Buck3(i) = (factor * ((t / b) ^ 2)) * I(i) / y;

end

elseif BMD(i) > 0 % if moment positive, compression on top

t = wt(z);

y = heights(i) - Y_bar(i);

b = abs(y) - tft(z);

if b ~= 0

M_Buck3(i) = (factor * ((t / b) ^ 2)) * I(i) / y;

end

end

end

end

end

end

function [ Pf, failure_mode ] = FailLoad( P, SFD, BMD, V_Mat, V_Buck, M_MatT, M_MatC, M_Buck1, M_Buck2, M_Buck3, V_GlueTF, V_GlueBF, V_GlueTW, V_GlueBW, write_to_xls, design_name )

% Calculates the magnitude of the load P that will cause one of the failure mechanisms to occur as well as the failure mode, and outputs all failure load value to excel sheet

%   Input: SFD, BMD under the currently applied points loads (P) (each 1-D array of length n)

%       {V_Mat, V_Glue, … M_MatT, M_MatC, … } (each 1-D array of length n), current load P, write_to_xls (a boolean value) and design_name (a string)

%   Output: Failure Load value Pf, and mode of failure (failure_mode) and writes to an excel sheet (optional) if write_to_xls is true

% Calculates failure loads

SFD_new = SFD ./ abs(P);

BMD_new = BMD ./ abs(P);

fail_v_mat = V_Mat ./ SFD_new;

fail_v_buck = V_Buck ./ SFD_new;

fail_m_matt = M_MatT ./ BMD_new;

fail_m_matc = M_MatC ./ BMD_new;

fail_m_buck1 = M_Buck1 ./ BMD_new;

fail_m_buck2 = M_Buck2 ./ BMD_new;

fail_m_buck3 = M_Buck3 ./ BMD_new;

fail_v_gluetf = V_GlueTF ./ SFD_new;

fail_v_gluebf = V_GlueBF ./ SFD_new;

fail_v_gluetw = V_GlueTW ./ SFD_new;

fail_v_gluebw = V_GlueBW ./ SFD_new;

% Finds lowest failure load for each array

matt = min(fail_m_matt(fail_m_matt>0));

matc = min(fail_m_matc(fail_m_matc>0));

buck1 = min(fail_m_buck1(fail_m_buck1>0));

buck2 = min(fail_m_buck2(fail_m_buck2>0));

buck3 = min(fail_m_buck3(fail_m_buck3>0));

vmat = min(fail_v_mat(fail_v_mat>0));

vbuck = min(fail_v_buck(fail_v_buck>0));

vgluetf = min(fail_v_gluetf(fail_v_gluetf>0));

vgluebf = min(fail_v_gluebf(fail_v_gluebf>0));

vgluetw = min(fail_v_gluetw(fail_v_gluetw>0));

vgluebw = min(fail_v_gluebw(fail_v_gluebw>0));

% Optionally write to .xls file if write_to_xls is true

if write_to_xls == true

filename = design_name + '.xls';

writematrix(design_name, filename);

writematrix("Moment Matboard Tension Failure", filename, 'range', 'A2')

writematrix(matt, filename, 'range', 'B2:C2');

writematrix("Moment Matboard Compression Failure", filename, 'range', 'A3')

writematrix(matc, filename, 'range', 'B3:C3');

writematrix("Case 1 Buckling of Top or Bottom Flange (center part between webs)", filename, 'range', 'A4')

writematrix(buck1, filename, 'range', 'B4:C4');

writematrix("Case 2 Buckling of Top or Bottom Flange (part that sticks out past the webs)", filename, 'range', 'A5')

writematrix(buck2, filename, 'range', 'B5:C5');

writematrix("Case 3 Buckling of Left and Right Webs", filename, 'range', 'A6')

writematrix(buck3, filename, 'range', 'B6:C6');

writematrix("Matboard Shear Failure", filename, 'range', 'A7')

writematrix(vmat, filename, 'range', 'B7:C7');

writematrix("Matboard Shear Buckling Failure", filename, 'range', 'A8')

writematrix(vbuck, filename, 'range', 'B8:C8');

writematrix("Shear Glue Failure of Top Flange", filename, 'range', 'A9')

writematrix(vgluetf, filename, 'range', 'B9:C9');

writematrix("Shear Glue Failure of Bottom Flange", filename, 'range', 'A10')

writematrix(vgluebf, filename, 'range', 'B10:C10');

writematrix("Shear Glue Failure of Top Flange/Web Connection", filename, 'range', 'A11')

writematrix(vgluetw, filename, 'range', 'B11:C11');

writematrix("Shear Glue Failure of Bottom Flange/Web Connection", filename, 'range', 'A12')

writematrix(vgluebw, filename, 'range', 'B12:C12');

end

% Outputs failure load and failure condition

results = ["Tension Failure", "Compression Failure", "Center Flange Buckling", "Protruding Flange Buckling", "Web Buckling", "V material", "vbuck", "Glue Shear tf", "Glue Shear bottom", "glue shear top width", "glue shear bottom width"];

[Pf, fail_index] = min([abs(matt) abs(matc) abs(buck1) abs(buck2) abs(buck3) abs(vmat) abs(vbuck) abs(vgluetf) abs(vgluebf) abs(vgluetw) abs(vgluebw)]);

% if fail_index = 1: matt, 2: matc, 3: buck1, 4: buck2, 5: buck3, 6: vmat, 7: vbuck, 8: vgluetf, 9: vgluebf, 10: vgluetw, 11: vgluebw

failure_mode = results(fail_index);

end

function [material_ok] = MaterialCheck( xc, tfw, tft, wh, wt, ws, bfw, bft, rw, rh, rbw, rbh)

% Checks if current design overuses material. Note does not take into account diaphragms and uses factor of safety of 1.1.

%   Input: cross-sectional dimensions

%   Output: matrial_ok (true or false) based on above conditions.

matboard_thickness = 1.27;

top = tfw .* tft;

top_glues = 2 * rh .* rw;

bottom_glues = 2 * rbh .* rbw;

webs = 2 * wh .* wt;

bottom = bfw .* bft;

total_cross_section_width = (top + top_glues + bottom_glues + webs + bottom)/matboard_thickness;

safety_factor = .9;

total_area  = 0;

for i = 2:length(xc)

delta_x = xc(i) - xc(i - 1);

total_area  = total_area + delta_x * total_cross_section_width(i);

end

total_material_area = 813 * 1016;

material_ok = total_area < total_material_area * safety_factor;

end

function [ V_GlueTF V_GlueBF V_GlueTW V_GlueBW ] = VglueFail(I, Q, b, TauG, tft, bft, heights, csc)

% Calculates glue failures for top flange, bottom flange and top and bottom web/flange joints

%   Input: Sectional properties (I, Q, b, sectional_height, etc), flange dimensions, cross-section changes, etc...

%   Output: V_GlueTF (top flange glue failure), V_GlueBF (bottom flange glue failure), V_GlueTW (Top flange/web joint glue failure)

%           V_GlueBW (bottom flange/web joint glue failure) all arrays of length n.

V_GlueBF = NaN(1,length(heights));

V_GlueTF = NaN(1,length(heights));

V_GlueTW = NaN(1,length(heights));

V_GlueBW = NaN(1,length(heights));

for i = 1 : length(heights)

z = find(csc <= i, 1, 'last');

% Top Flange Glue

if (tft(z) / 1.27) > 1

glue_y = ceil(heights(i) - 1.27);

V_GlueTF(i) = (TauG * b(i, glue_y) * I(i)) / ((Q(i, floor(heights(i) - 1.27)) + Q(i, ceil(heights(i) - 1.27))) / 2);

end

% Bottom Flange Glue

if (bft(z) / 1.27) > 1

glue_y = floor(1.27);

V_GlueBF(i) = (TauG * b(i, glue_y) * I(i)) / ((Q(i, floor(1.27)) + Q(i, ceil(1.27)))/2);

end

% Top web/flange glue (there will always be a top flange therefore no if statement needed)

%   (note: rounds up heights(i) - tft(i) in order to get smaller b value and larger Q value in order to get lower V_GlueTW for safety

V_GlueTW(i) = (TauG * b(i, floor(heights(i) - tft(z))) * I(i)) / ((Q(i, floor(heights(i) - tft(z))) + Q(i, ceil(heights(i) - tft(z)))) / 2);

% Bottom web/flange glue (note: rounds up bft(i) in order to get smaller b value and larger Q value in order to get lower V_GlueBW for safety

if bft(z) > 0

V_GlueBW(i) = (TauG * b(i, ceil(bft(z))) * I(i)) / ((Q(i, floor(bft(z))) + Q(i, ceil(bft(z))))/2);

end

end

end